| Primary Tasks – Hazards | Primary Tasks - Controls |
| --- | --- |
| Noise from process equipment | Wear hearing protection |
| Congested areas | Avoid congestion, if possible, communicate with others if not |
| Dust (breathing or in eyes) | Dust mask optional, glasses mandatory |
| Heaving awkward lifting | Get help, lift properly, use machines where possible |
| Loose Materials | Keep area organized |
| Scaffolds, Ladders, Stair Use | Read tags, 3-point contact |
| Vehicles and Equipment | Make eye contact with operators |

| Specific pinch, crush point | Specific pinch, crush point |
| --- | --- |
| Hazards | Controls |
| Tools, Tighteners, Nippers | Watch hand placement, while wearing gloves |
| Pinch points in workspace | Wear appropriate gloves |
| Metal bands, saws, knives | Watch hand placement, while wearing gloves |
| Specific Slip/Trip | Specific Slip/Trip |
| Hazards | Controls |
| Ice & Mud (Slips) | Wear traction aids on icy surfaces |
| Curbs, Dunnage and Uneven ground | Stick to main paths, watch footing |
| Specific Falling Objects | Specific Falling Objects |
| Hazards | Controls |
| Falling Tools | Use tool retention at heights (mandatory) |
| Hard hat knock or blow off head | Wear and use hard hat lanyards |
| Dust from cutting insulation | Cut over a bag or drop sheet |

1. Transfer the gore settings from the gore ruler as shown in fig. 1

2. Use the square to draw 90-degree lines through each mark, lines should be at least 1" longer than the 1/2 the heel measurement. See figure 2.

3. Line 1 is the throat line. Add 3/8" for the beat to the throat measurement. See figure 3.

4. Line #9 is your heel line. Add 3/4" for the crimp to your heel measurement and then divide by 2. Transfer the length to line #9. Example 6" Heel + 3/4" crimp = 6-3/4". Divide this by 2 to get 3-3/8". Transfer this to line 9.

5. Find your circumference (3.142 X Diameter) and draw a line this length. Divide this line into 16 equal parts. Now we transfer the lengths from step 4 to our circumference line as shown below. Now the gore pattern is complete. See figure 4

1. Measure outer diameter of pipe, divided by 2 to get radius. 2. Locate the radius on the T-Master (between A&B). 3. While holding the T-Master steady in one position, trace the curve at this radius and transfer the 4. squaring tabs as shown on the reverse. 5. Remove the T-Master and connect the squaring tab marks, this will give you your 90 lines.